# Project Activity Log

Clayton Thomas McIntyre
April 3, 2025

# New Cognex Camera

Emailed/Called Joey Bradshaw multiple times about concern with GPU requirements for camera.

- The Vidi Software for training the AI model will need an NVIDIA GeForce RTX 30 Series (3090/3080/3070) driver greater than or equal to version 457.09. According to Joey, we do not need it on the computers during runtime, just for training. Provided that the camera works for us we will need to acquire hardware if we want to do the programming in-house.

- Chuck did not buy a new GPU, the 30 series GPU’s are old and are only available from resellers. Verified with Joey Bradshaw that a newer, more readily available GPU will work.

- We won’t request the purchase of a new GPU until we confirm the camera will work.

# Welder 3

Harry is supposed to follow up regarding an issue with run-aways on seam welds. I sent him log files and he has not investigated them yet, I’ve continued to bring it up to him.

Gave Harry more information regarding when the runaways happened, last we talked about it he said he had investigated it some but had not found a solution yet.

Assisted Luis with tuning the new motors.

Helped Ralph route the wires for the new camera.

Crunched the numbers to determine how much faster the new motors are

# Welder 4

Fixed issue with Top Side# 68 welds coming out light

Fixed issue with Live View(non-inspection) Light setting on Side 3 tabs, the lights were too dark for operator to do manual welds.

Adjusted both fixture A and fixture B side 3 tabs to reduce manual welds.

Obtained video evidence that the Y-axis is shifting between runs, see screenshots on the last two pages. Tested it both with gas on and off, saw issue in both scenarios.

Very few manual welds.

Continuous monitoring of welds, adjusting as needed.

Assisted in removal of stripped screw, resulting in a damaged spacer. Working with production to prevent this issue in the future since it’s happened more than once.

# WeldBuddy

Have a basic buildout made that is accessible through a web browser, current functionalities: I can initiate a job, set part inventories, weld spacers, and there is text-based weld logging.

Can now write weld and scrap logs to a database

I think I have set it up to do multiple sessions, but I can’t test it until we have it on a server. Chuck said that we may be able to set me up on a virtual machine to test it, we will meet about it either tomorrow or next week.

Met with Ryan and Carlos to refine scope, will set up meeting with larger group once scope finalized.

Made it so that software can handle multiple sessions simultaneously

Added export function so that logs can be downloaded as excel reports

Fixed a few bugs

# Mod-C Fixture Comparison

I have the Mod-C fixtures from welder 3. I am going to measure them and see if we can find discrepancies between them that could explain the unbonded t-welds we continuously have on Fixture B.

There are some differences between the fixtures that I believe are causing our buckling and T-Weld issues. See Report of findings on page 3. Credit to Angela in QA for teaching me how to use the Keyence machine.

I need to have a discussion with the group to discuss how we will resolve this.

# Air Knife Prototypes

Assisted Eric in testing prototypes.

Assisted with 3D printing.

Redid testing under higher pressure and with more versions of air knives, Eric will present findings.

# Triton

Measured distance between overlap to radius for all A Cells on an assembled Triton spacer to determine how much more of an overlap we can get on the A Cells.

# Training

Continued onboarding