# Project Activity Log

Clayton Thomas McIntyre
August 21, 2025

# Welder 3

Inspected spacers of second shift laser operator prior to welding

Set up production run for Triton Bottoms

Adjusted address issue with burndowns around the water rods

Created new weld routines with “reversed racetrack” to reduce probability of burndowns

Provided assembly tips to production to reduce number of gaps to avoid burndowns

Reported issue with welder pausing during welding

Reset global offsets after Monday’s incident with the camera, Majority of my time Monday and Tuesday

Adjusted welds to fix frame/cell rejects

As of yesterday am no longer inspecting second shifts spacers prior to welding

# WeldBuddy and Weld Stats

Added charts to welding statistics

Added automated saving of weld statistics

Coming early tomorrow to work with IT to debug issues with connecting to database on server

Was able to fix issues with database accessibility with IT

Program is now accessible from any computer at Caran; however it is not functioning on the server and needs further debugging which I have not had time to do

# Welder 4

Investigated runaway seam welds, discovered clue in log files that lead to a possible solution

Implemented potential solution

# Welder 2

Began setting “Global” positioning for GNF3 lower spacer so functional testing can begin

With guidance from Luis, tuned to motors to fix issue with vibrations

Figured out what the settings need to be for perpendicularity of the saddle

Assembled a spacer so we can finish setting the globals

Assisted Harry with getting lights functioning within the chamber

# Smart Scope Inspection Recording

Assigned project

In communication with vendor to supply a system to record inspections

Met with vendor to discuss system

Caran will borrow a “loaner” PC from the vendor, and purchase some hardware to begin functional testing to ensure we are able to capture video feed before purchasing a full system

# Triton Overlay Requirement

Obtained Drawings and final inspection reports of the current parts being used

Requested final inspection reports for previous parts