# Project Activity Log

Clayton Thomas McIntyre
October 30, 2025

# Triton Envelope Issue

Finished bagging and tagging posts in preparation for test

Performed first test with 0.001” larger posts on perimeter, results pending

Both test spacers have been welded and submitted to Quality, they should be heat treated next week

# Welder 3

No Machine Errors since last Wednesday!

Unable to work with Luck on gap detection, schedules haven’t aligned

Added gap detection to two more inspection types on cell-to-cell, after welding a test spacer we had excessive manual welds so I reverted to the previous version of the inspection routines without the cell-to-cell gap detection

Only Cell-to-frame welds have gap detection

Will resume adding gap detection next time we run Triton, the gap detection on the cell-to-cell work, it just needs to be optimized so we aren’t getting excessive manual welds, we’ll be able to do that on the next production run

Set the perpendicularity of the saddle to the optics tube, or “leveled” the saddle

Installed the old camera

Confirmed that the correlation on the old camera is still accurate

# Welder 2

Fixed issue with camera inspection routines

Assisted Harry as needed with dead reckoning

Assisted Eric in installing new camera mount

Lined up laser with camera

Redid all the globals

Began Weld Development on GNF3 Lower

Top Side single-single welds optimized

Currently working on bottom side single-single welds

# Welder 4

Bug fixed with lighting, will be able to continue with backlighting in the coming weeks

No Update

# Welder 6

Squared up weld chamber

Assisted Eric in installing new camera mount

Correlated Camera

Showed Philip how to correlate camera

Set globals for GNF3 Upper

Tweaked some of the inspection routine settings so they behave better

# WeldBuddy and Weld Stats

IT installed service on welder 4 to add idle timer to Weldbuddy server

Harry needs to move where the idle timer is saving so that it can upload to server

Weld statistics page in its preliminary setup, statistics seem to be calculating correctly, check it out WeldBuddy — Log Analysis

Dual monitor mount purchased

Request submitted to Maintenance to install mount

After mount is installed IT will hook up second monitor and we will begin beta testing during LTZ for Weldbuddy