# Project Activity Log

Clayton Thomas McIntyre
November 06, 2025

# Triton Envelope Issue

Waiting for Heat Treated Spacers

# Welder 3

Added gap detection to two more inspection types on cell-to-cell, after welding a test spacer we had excessive manual welds so I reverted to the previous version of the inspection routines without the cell-to-cell gap detection

Only Cell-to-frame welds have gap detection

Will resume adding gap detection next time we run Triton, the gap detection on the cell-to-cell work, it just needs to be optimized so we aren’t getting excessive manual welds, we’ll be able to do that on the next production run

Set the perpendicularity of the saddle to the optics tube, or “leveled” the saddle

Installed the old camera

Confirmed that the correlation on the old camera is still accurate

Worked with Harry to troubleshoot memory issues

I had to revert to an old version of the interface to get the weld program to compile

Attempted to improve results for inspection routines for seam and tab welds, and was unable to get reliable results

Made a report with some ideas to share with Luck, he will have to implement them because I don’t know how to make new rules yet

# Welder 2

Was able to shave off ~4 seconds per tab weld, will result in about 5 minutes faster cycle time on GNF3 spacers

Having roll back issues on seam welds that I’m trying to fix

Been working closely with Evan, showing him all the parameters we’re able to adjust

Single to Single, Double to Single, and tab welds are dialed in

Still need to get seam, bottom edge, Double to double, and plate welds optimized

# WeldBuddy and Weld Stats

Dual monitor mount purchased

Request submitted to Maintenance to install mount

After mount is installed IT will hook up second monitor and we will begin beta testing during LTZ for Weldbuddy

Dual monitors and second monitor installed, Weldbuddy ready to be tested on our next production run

Need to set up LTZ spacer in Weldbuddy, need part list with # of parts per spacer

# Miscellaneous

I’ve been shutting the gas off to the welders at the wall at night to save argon